## PP1 Vsensor V4 – Rx Supply Voltage swapped polarity fix

- Break VSSA Net under FB201

  - Remove FB201

  - break trace to via

  - Re-Assemble FB201

- Break VDDA trace to U701

- Break VDDA branch to U704 and U702

- Connect supply branch U701, U702 and U704

- Connect Pin1 C717 (old VDDA) to Pin1 C2012 (VSSA)

- Connect Pin2 C716 (old VSSA) to Pin1 C226 (VDDA)